/****************************************************************************

Module

Navigating.c

Revision

2.0.1

Description

This is a template file for implementing state machines.

Notes

History

When           Who     What/Why

-------------- ---     --------

02/07/13 21:00 jec      corrections to return variable (should have been

ReturnEvent, not CurrentEvent) and several EV_xxx

event names that were left over from the old version

02/08/12 09:56 jec      revisions for the Events and Services Framework Gen2

02/13/10 14:29 jec      revised Start and run to add new kind of entry function

to make implementing history entry cleaner

02/13/10 12:29 jec      added NewEvent local variable to During function and

comments about using either it or Event as the return

02/11/10 15:54 jec      more revised comments, removing last comment in during

function that belongs in the run function

02/09/10 17:21 jec      updated comments about internal transitions on During funtion

02/18/09 10:14 jec      removed redundant call to RunLowerlevelSM in EV_Entry

processing in During function

02/20/07 21:37 jec      converted to use enumerated type for events & states

02/13/05 19:38 jec      added support for self-transitions, reworked

to eliminate repeated transition code

02/11/05 16:54 jec      converted to implment hierarchy explicitly

02/25/03 10:32 jec      converted to take a passed event parameter

02/18/99 10:19 jec      built template from MasterMachine.c

02/14/99 10:34 jec      Began Coding

****************************************************************************/

/*----------------------------- Include Files -----------------------------*/

// Basic includes for a program using the Events and Services Framework

#include "ES_Configure.h"

#include "ES_Framework.h"

#include "ES_DeferRecall.h"

#include "TestHarnessService0.h"

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "inc/hw_gpio.h"

#include "inc/hw_sysctl.h"

#include "driverlib/sysctl.h"

#include "driverlib/pin_map.h"	// Define PART_TM4C123GH6PM in project

#include "driverlib/gpio.h"

#include "ES_ShortTimer.h"

#include "inc/hw_pwm.h"

#include "inc/hw_timer.h"

#include "inc/hw_nvic.h"

#include <stdlib.h>

/* include header files for this state machine as well as any machines at the

next lower level in the hierarchy that are sub-machines to this machine

*/

#include "Master.h"

#include "MapKeys.h"

#include "MotorDrive.h"

#include "IRBeacon.h"

#include "Flywheel.h"

#include "LaunchManager.h"

#include "SPI.h"

#include "LimitSwitch.h"

#include "Define.h"

#include "Staging.h"

#include "Reloading.h"

#include "Shooting.h"

#include "BackToTape.h"

#include "Navigating.h"

#include "Tape.h"

#include "LEDIndicators.h"

/*----------------------------- Module Defines ----------------------------*/

// define constants for the states for this machine

// and any other local defines

#define ENTRY_STATE STATE_WaitingToStart

/*---------------------------- Module Functions ---------------------------*/

/* prototypes for private functions for this machine, things like during

functions, entry & exit functions.They should be functions relevant to the

behavior of this state machine

*/

static ES_Event DuringWaitingToStart( ES_Event Event);

static ES_Event DuringQueryStagingArea( ES_Event Event);

static ES_Event DuringCheckNextStep( ES_Event Event);

/*---------------------------- Module Variables ---------------------------*/

// everybody needs a state variable, you may need others as well

static NavigatingState_t CurrentState;

static uint8_t  GameStatus;

static uint32_t Response;

static uint8_t BallCount;

static uint8_t CurrentStage;

static bool Team;

static bool CS;

/*------------------------------ Module Code ------------------------------*/

/****************************************************************************

Function

RunTemplateSM

Parameters

ES_Event: the event to process

Returns

ES_Event: an event to return

Description

add your description here

Notes

uses nested switch/case to implement the machine.

Author

J. Edward Carryer, 2/11/05, 10:45AM

****************************************************************************/

ES_Event RunNavigating( ES_Event CurrentEvent )

{

bool MakeTransition = false;/* are we making a state transition? */

NavigatingState_t NextState = CurrentState;

ES_Event EntryEventKind = { ES_ENTRY, 0 };// default to normal entry to new state

ES_Event ReturnEvent = CurrentEvent; // assume we are not consuming event

ES_Event PostEvent;

switch ( CurrentState )

{

case STATE_WaitingToStart :       // If current state is state one

// Execute During function for state one. ES_ENTRY & ES_EXIT are

// processed here allow the lower level state machines to re-map

// or consume the event

CurrentEvent = DuringWaitingToStart(CurrentEvent);

//process any events

if ( CurrentEvent.EventType != ES_NO_EVENT ) //If an event is active

{

printf("Navigating WaitingToStart \r\n");

switch (CurrentEvent.EventType)

{

case ES_RESPONSE_READY : //If event is event one

// Execute action function for state one : event one

//get response

Response = GetResponse();

//check game status, Status Byte 3 bit7

GameStatus = ((uint8_t)(Response>>(8*0)) & BIT7HI);

printf("game status %u \r\n", GameStatus);

//if game start

if(GameStatus == Construction_Active){

//Start game timer 2:18 = 138s * 976 (one sec)

InitGameTimer();

PostEvent.EventType = GAME_START;

PostLEDIndicators(PostEvent); //turn on construction in progress LED

NextState = STATE_QueryStagingArea;//Decide what the next state will be

// for internal transitions, skip changing MakeTransition

MakeTransition = true; //mark that we are taking a transition

// if transitioning to a state with history change kind of entry

// optionally, consume or re-map this event for the upper

// level state machine

ReturnEvent.EventType = ES_NO_EVENT;

}else{

//query again

PostEvent.EventType = ES_SEND_COMMAND;

PostEvent.EventParam = Status;

PostSPI(PostEvent);

// optionally, consume or re-map this event for the upper

// level state machine

ReturnEvent.EventType = ES_NO_EVENT;

}

break;

// repeat cases as required for relevant events

}

}

break;

// repeat state pattern as required for other states

case STATE_QueryStagingArea :       // If current state is state one

// Execute During function for state one. ES_ENTRY & ES_EXIT are

// processed here allow the lower level state machines to re-map

// or consume the event

CurrentEvent = DuringQueryStagingArea(CurrentEvent);

//process any events

if ( CurrentEvent.EventType != ES_NO_EVENT ) //If an event is active

{

printf("Navigating QueryStagingArea \r\n");

switch (CurrentEvent.EventType)

{

case ES_CHECK_NEXT_STEP : //If event is event one

// Execute action function for state one : event one

NextState = STATE_CheckNextStep;//Decide what the next state will be

// for internal transitions, skip changing MakeTransition

MakeTransition = true; //mark that we are taking a transition

// if transitioning to a state with history change kind of entry

// optionally, consume or re-map this event for the upper

// level state machine

ReturnEvent.EventType = ES_NO_EVENT;

break;

// repeat cases as required for relevant events

case ES_RESPONSE_READY : //If event is event one

// Execute action function for state one : event one

Team = GetTeam();

switch(Team){

case GREEN:

//get response

Response = GetResponse();

//if it's ready for Green to check in

//CSG is Status Byte 1 bit7

CS = ((uint8_t)(Response>>(8*2)) & BIT7HI);

break;

case RED:

//get response

Response = GetResponse();

//if it's ready for Red to check in

//CSR is Status Byte 1 bit3

CS = ((uint8_t)Response>>(8*2) & BIT3HI);

break;

}

if(CS != 0){//ask again

MakeTransition = true; //mark that we are taking a transition

// if transitioning to a state with history change kind of entry

// optionally, consume or re-map this event for the upper

// level state machine

ReturnEvent.EventType = ES_NO_EVENT;

}

}

}

break;

// repeat state pattern as required for other states

case STATE_CheckNextStep :       // If current state is state one

// Execute During function for state one. ES_ENTRY & ES_EXIT are

// processed here allow the lower level state machines to re-map

// or consume the event

CurrentEvent = DuringCheckNextStep(CurrentEvent);

//process any events

if ( CurrentEvent.EventType != ES_NO_EVENT ) //If an event is active

{

printf("Navigating CheckNextStep \r\n");

switch (CurrentEvent.EventType)

{

case ES_NEW_ROUND : //If event is event one

// Execute action function for state one : event one

NextState = STATE_QueryStagingArea;//Decide what the next state will be

// for internal transitions, skip changing MakeTransition

MakeTransition = true; //mark that we are taking a transition

// if transitioning to a state with history change kind of entry

// optionally, consume or re-map this event for the upper

// level state machine

ReturnEvent.EventType = ES_NO_EVENT;

break;

// repeat cases as required for relevant events

}

}

break;

// repeat state pattern as required for other states

}

//   If we are making a state transition

if (MakeTransition == true)

{

//   Execute exit function for current state

CurrentEvent.EventType = ES_EXIT;

RunNavigating(CurrentEvent);

CurrentState = NextState; //Modify state variable

//   Execute entry function for new state

// this defaults to ES_ENTRY

RunNavigating(EntryEventKind);

}

return(ReturnEvent);

}

/****************************************************************************

Function

StartTemplateSM

Parameters

None

Returns

None

Description

Does any required initialization for this state machine

Notes

Author

J. Edward Carryer, 2/18/99, 10:38AM

****************************************************************************/

void StartNavigating ( ES_Event CurrentEvent )

{

// to implement entry to a history state or directly to a substate

// you can modify the initialization of the CurrentState variable

// otherwise just start in the entry state every time the state machine

// is started

if ( ES_ENTRY_HISTORY != CurrentEvent.EventType )

{

CurrentState = ENTRY_STATE;

}

// call the entry function (if any) for the ENTRY_STATE

RunNavigating(CurrentEvent);

}

/****************************************************************************

Function

QueryTemplateSM

Parameters

None

Returns

TemplateState_t The current state of the Template state machine

Description

returns the current state of the Template state machine

Notes

Author

J. Edward Carryer, 2/11/05, 10:38AM

****************************************************************************/

NavigatingState_t QueryNavigating ( void )

{

return(CurrentState);

}

/***************************************************************************

private functions

***************************************************************************/

static ES_Event DuringWaitingToStart( ES_Event Event)

{

ES_Event ReturnEvent = Event; // assme no re-mapping or comsumption

ES_Event PostEvent;

// process ES_ENTRY, ES_ENTRY_HISTORY & ES_EXIT events

if ( (Event.EventType == ES_ENTRY) ||

(Event.EventType == ES_ENTRY_HISTORY) )

{

// implement any entry actions required for this state machine

//query game status

PostEvent.EventType = ES_SEND_COMMAND;

PostEvent.EventParam = Status;

PostSPI(PostEvent);

// after that start any lower level machines that run in this state

//StartLowerLevelSM( Event );

// repeat the StartxxxSM() functions for concurrent state machines

// on the lower level

}

else if ( Event.EventType == ES_EXIT )

{

// on exit, give the lower levels a chance to clean up first

//RunLowerLevelSM(Event);

// repeat for any concurrently running state machines

// now do any local exit functionality

}else

// do the 'during' function for this state

{

// run any lower level state machine

// ReturnEvent = RunLowerLevelSM(Event);

// repeat for any concurrent lower level machines

// do any activity that is repeated as long as we are in this state

}

// return either Event, if you don't want to allow the lower level machine

// to remap the current event, or ReturnEvent if you do want to allow it.

return(ReturnEvent);

}

static ES_Event DuringQueryStagingArea( ES_Event Event)

{

ES_Event ReturnEvent = Event; // assme no re-mapping or comsumption

ES_Event PostEvent;

// process ES_ENTRY, ES_ENTRY_HISTORY & ES_EXIT events

if ( (Event.EventType == ES_ENTRY) ||

(Event.EventType == ES_ENTRY_HISTORY) )

{

// implement any entry actions required for this state machine

//query staging area

PostEvent.EventType = ES_SEND_COMMAND;

PostEvent.EventParam = Status;

PostSPI(PostEvent);

// after that start any lower level machines that run in this state

//StartLowerLevelSM( Event );

// repeat the StartxxxSM() functions for concurrent state machines

// on the lower level

}

else if ( Event.EventType == ES_EXIT )

{

// on exit, give the lower levels a chance to clean up first

//RunLowerLevelSM(Event);

// repeat for any concurrently running state machines

// now do any local exit functionality

}else

// do the 'during' function for this state

{

// run any lower level state machine

// ReturnEvent = RunLowerLevelSM(Event);

// repeat for any concurrent lower level machines

// do any activity that is repeated as long as we are in this state

}

// return either Event, if you don't want to allow the lower level machine

// to remap the current event, or ReturnEvent if you do want to allow it.

return(ReturnEvent);

}

static ES_Event DuringCheckNextStep( ES_Event Event)

{

ES_Event ReturnEvent = Event; // assme no re-mapping or comsumption

ES_Event PostEvent;

// process ES_ENTRY, ES_ENTRY_HISTORY & ES_EXIT events

if ( (Event.EventType == ES_ENTRY) ||

(Event.EventType == ES_ENTRY_HISTORY) )

{

// implement any entry actions required for this state machine

//check ball count

BallCount = GetBallCount();

//check CurrentLocation

CurrentStage = GetLocation();

//if current stage is stage 1 & ball count < 5, reload

if((CurrentStage == Stage_One) && (BallCount<3)){

PostEvent.EventType = ES_RELOAD;

PostMasterSM(PostEvent);

}

//else if empty, reload

else if(BallCount == 0){

PostEvent.EventType = ES_RELOAD;

PostMasterSM(PostEvent);

}else{//else

PostEvent.EventType = ES_NEW_ROUND;

PostMasterSM(PostEvent);

}

// after that start any lower level machines that run in this state

//StartLowerLevelSM( Event );

// repeat the StartxxxSM() functions for concurrent state machines

// on the lower level

}

else if ( Event.EventType == ES_EXIT )

{

// on exit, give the lower levels a chance to clean up first

//RunLowerLevelSM(Event);

// repeat for any concurrently running state machines

// now do any local exit functionality

}else

// do the 'during' function for this state

{

// run any lower level state machine

// ReturnEvent = RunLowerLevelSM(Event);

// repeat for any concurrent lower level machines

// do any activity that is repeated as long as we are in this state

}

// return either Event, if you don't want to allow the lower level machine

// to remap the current event, or ReturnEvent if you do want to allow it.

return(ReturnEvent);

}